Name : S Hari Prasath

REG NO : 192512041

EX NO: 1 – Butterworth IIR Filters

DATE: 23/07/2026

1.1. Design and Analysis of a Butterworth IIR Low-Pass Filter Using MATLAB.

Objectives

- To determine the minimum filter order and cutoff frequency, required to satisfy the design constraints.

- To design and analyze the magnitude response, phase response, characteristics of the Butterworth IIR low-pass filter for the given passband and stopband specifications.

- To analyze the impulse response, step response and group delay characteristics of the Butterworth IIR low-pass filter for the given passband and stopband specifications.

- To examine the pole-zero plot and verify the stability and performance of the designed low-pass filter.

Procedure

- Open MATLAB and create a new script file in the MATLAB Editor.

- Define the filter specifications such as passband ripple (Rp), stopband attenuation (Rs), passband frequency (Fp), stopband frequency (Fsb), and sampling frequency (Fs).

- Calculate the normalized passband and stopband frequencies with respect to the Nyquist frequency (Fs/2).

Program Code:

clc;

clear;

close all;

Fs = 10000;

Fp = 1000;

Fst = 1500;

[N,Wn] = buttord(Fp/(Fs/2),Fst/(Fs/2),1,60);

[b,a] = butter(N,Wn);

% Frequency Response

figure;

freqz(b,a);

% Impulse Response

[h,n] = impz(b,a,50);

figure;

stem(n,h,'filled');

grid on;

xlabel('Samples');

ylabel('Amplitude');

title('Impulse Response');

% Step Response

figure;

stepz(b,a);

% Pole-Zero Plot

figure;

zplane(b,a);

fprintf('Minimum Filter Order (N) = %d\n',N);

fprintf('Cutoff Frequency (Wn) = %.4f\n',Wn);

OUTPUT AND RESULT:

Fig 4

Fig 3

Fig 2

Fig 1

Result:

1.The minimum filter order and cutoff frequency satisfying the given specifications were

determined successfully.

2.The magnitude and phase responses of the Butterworth IIR low-pass filter were obtained and

analyzed.

3.The impulse response, step response, and group delay characteristics were evaluated

successfully.

4. The pole-zero plot confirmed the stability and proper performance of the designed filter.